Gansu Winshine Metallurgy Chemical Co.,Ltd.

| E-mail: gansuwinshine@126.com | Tel:0086-931-8840552 |
| --- | --- |
| Address:405 Gannan Road,Lanzhou,Gansu,China | Fax:0086-931-8840553 |

# Material Safety Data Sheet

##### 1.IDENTIFICATION OF THE MATERIAL AND SUPPLIER

Product   Name:

Other   name(s):

Recommended  use  of  the  chemical and  restrictions  on  use:

Supplier:

Street   Address:

Email:

Telephone  Number:

Facsimile:

Emergency Telephone:

2.HAZARDS                      IDENTIFICATION

SODIUM   ISOBUTYL   XANTHATE

SIBX;Carbonodithioic   acid,O-(2-methylpropyl)ester,sodium    salt. Mineral floatation.

GANSU WINSHINE METALLURGY CHEMICALS CO.,LTD NO.405 GANNAN ROAD,LANZHOU,GANSU,CHINA

gansuwinshine@126.com

+86-931-8840552

+86-931-8840553  119(ALL  HOURS)

Classified  as  Dangerous  Goods  by  the  criteria  of  the  Chinese  Dangerous  Goods  Code(CDG  Code)for  Transport  by  Road and    Rail;DANGEROUS    GOODS.

This  material  is  hazardous  according  to  Safe  Work  Chinese;HAZARDOUS  SUBSTANCE.

Self-heating substances and mixtures-Category  1

Classification of the substance or mixture:

Acute  Oral  Toxicity-Category  4

Acute  Dermal  Toxicity-Category  4

Skin  Irritation-Category  2

Eye  Irritation-Category  2A

SIGNAL  WORD:DANGER

Hazard  Statement(s):

H251 Self-heating;may catch fire.

H302+H312 Harmful if swallowed or in contact with skin.

H319 Causes serious eye iritation.

H315 Causes skin irritation.

Precautionary Statement(s):

Prevention:

P235+P410 Keep cool.Protect from sunlight.

P264 Wash hands thoroughly after handling.

P270 Do not eat,drink or smoke when using this product.

P280 Wear protective gloves/protective clothing/eye protection/face protection.

Response:

P301+P312IF SWALLOWED:Call a POISON CENTER or doctor/physician if you feel unwell. P330 Rinse mouth.

P302+P352IF ON SKIN:Wash with plenty of soap and water.

P321 Specific treatment(see First Aid Measures on Safety Data Sheet).

P332+P313 If skin iritation occurs:Get medical advice/attention.

Gansu Winshine Metallurgy Chemical Co.,Ltd.

| E-mail:gansuwinshine@126.com | Tel:0086-931-8840552 |
| --- | --- |
| Address:405 Gannan Road,Lanzhou,Gansu,China | Fax:0086-931-8840553 |

P362 Take off contaminated clothing and wash before reuse.

P363Wash contaminated clothing before re-use.

P305+P351+P338 IF INEYES:Rinse cautiously with water for several minutes.Remove contact lenses,if present and easy to do. Continue rinsing.

P337+P313 If eye irritation persists:Get medical advice/attention.

P312 Call a POISON CENTER or doctor/physician if you feel unwell

P322 Specific measures (see First Aid Measures on Safety Data Sheet)

Storage:

P407 Maintain air gap between stacks/pallets.

P420 Store away from other materials.

Disposal:

P501 Dispose of contents/container in accordance with local/regional/nationalinternational regulations.

Poisons   Schedule(SUSMP):                              None  allocated

3.COMPOSITION/INFORMATION  ON  INGREDIENTS

Components                             CAS Number

Sodium isobutyl xanthate        25306-75-6

Free Alkali                                148262-19-5

Moisture&Volatile                     7732-18-5

Proportion

90%

2% 8%

Hazard Codes YES

NO

NO

##### 4.FIRST AID MEASURES

For advice,contact a Poisons Information Centre(e.g.phone China 120;or a doctor.

Inhalation:

Remove victim from area of exposure-avoid becoming a casualty.Remove contaminated clothing and loosen remaining

clothing.Alow  patient to assume  most comfortable  position and  keep warm.Keep at  rest  untiful  recovered.f patient  inds breathing dificult and develops a bluish discolouration of the skin(which suggests a lack of oxygen in the blood-cyanosis),   ensure airways are clear of any obstruction and have a qualified person give oxygen through a face mask.Apply artificial

respiration if patient is not breathing.Seek immediate medical advice.

Skin Contact:

If  skin  or   hair  contact  occurs,immediately   remove   any   contaminated   clothing  and  wash  skin  and   hair  thoroughly  with   running water.f    swelling,redness,blistering    or     iritation    occurs     seek     medical    assistance.

Eye Contact:

lf in eyes,hold eyelids apart and flush the eye continuously with running water.Continue flushing until advised to stop by a Poisons Information Centre or a doctor,or for at least 15 minutes.

Ingestion:

Rinse mouth with water.If swallowed,do NOT induce vomiting.Give a glass of water.Seek immediate medical assistance. Indication of immediate medical attention and special treatment needed:

Treat symptomatically.

##### 5.FIRE FIGHTING MEASURES

Suitable Extinguishing Media:

Coarse water spray,fine water spray,normal foam,dry agent (dry chemical powder).Use flooding amounts of water to stop the reaction;smothering will not work as this material does not need air to burn.

Hazchem or Emergency Action Code:1Y

Specific hazards arising from the substance or mixture:

Substance liable to spontaneous combustion.

Special protective equipment and precautions for fire-fighters:

Gansu Winshine Metallurgy Chemical Co.,Ltd.

E-mail:gansuwinshine@126.com                                                              Tel:0086-931-8840552

Address:405 Gannan Road,Lanzhou,Gansu,China                               Fax:0086-931-8840553

Avoid allignition sources.Heating can cause expansion or decomposition of the material,which can lead to the containers exploding.If safe to do so,remove containers from the path ofire.Decomposes on heating emitting toxic fumes,including those of oxides of sulfur .Fire fighters to wear self-contained breathing apparatus and suitable protective clothing if risk of  exposure to products of decomposition.

##### 6.ACCIDENTAL  RELEASE  MEASURES

Emergency procedures/Environmental precautions:

Shut off all possible sources of ignition.Clear area of all unprotected personnel.If contamination of sewers or waterways has occurred advise local emergency services.

Personal precautions/Protective equipment/Methods and materials for containment and cleaning up:

Wear protective equipment to prevent skin and eye contact and breathing in vapours/dust.Air-supplied masks are

recommended to avoid inhalation of toxic material.Vacuum solid spills instead of sweeping.Collect and seal in properly labelled containers or drums for disposal.Use non-sparking tools.

##### 7.HANDLING  AND   STORAGE

Precautions for safe handling:

Avoid skin and eye contact and breathing in dust.In common with many organic chemicals,may form flammable dust clouds in air.For precautions necessary refer to Safety Data Sheet "Dust Explosion Hazards".

Conditions for safe storage,including any incompatibilities:

Store in a cool,dry,well ventilated place and out of direct sunlight.Store away from sources of heat or ignition.Store away from incompatible materials described in Section 10.Keep dry-reacts with water,may lead to drum rupture.Keep

containers closed when not in use-check regularly for spills.

8.EXPOSURE    CONTROLS/PERSONAL    PROTECTION

Control Parameters:No value assigned for this specific material by Safe Work China.However,Workplace Exposure Standard(s)for  decomposition  product(s):

Carbon  disulfide:8hr  TWA=31   mg/m3(10  ppm),Sk

As published by Safe Work China Workplace Exposure Standards for Airborne Contaminants.

TWA-The time-weighted average airborne concentration of a particular substance when calculated over an eight-hour working day,for a five-day working week.

'Sk'(skin)Notice-absorption through the skin may be a significant source of exposure.The exposure standard is invalidated if such contact should occur.

These Workplace Exposure Standards are guides to be used in the control of occupational health hazards.All atmospheric contamination should be kept to as low a level as is workable.These workplace exposure standards should not be used as fine dividing lines between safe and dangerous concentrations of chemicals.They are not a measure of relative toxicity.

Appropriate engineering controls:

Ensure ventilation is adequate and that air concentrations of decomposition product(s)is/are controlled below quoted   Exposure Standards.Avoid generating and breathing in dusts.Use with local exhaust ventilation or while wearing dust mask.Keep containers closed when not in use.

Individual protection measures,such as Personal Protective Equipment(PPE):

The selection of PPE is dependant on a detailed risk assessment.The risk assessment should consider the work situation, the physical form of the chemical,the handling methods,and environmental factors.

Personal   Protection   Guide   No.1,1998:F-OVERALLS,SAFETY   SHOES,CHEMICAL   GOGGLES,GLOVES,DUST MASK.

Wear overalls,chemical goggles and impervious gloves.Avoid generating and inhaling dusts.If dust exists,wear dust

mask/respirator meeting the requirements of ASINZS 1715 and ASINZS 1716.Always wash hands before smoking,eating, drinking or using the toilet.Wash contaminated clothing and other protective equipment before storage or re-use.

Gansu Winshine Metallurgy Chemical Co.,Ltd.

| E-mail:gansuwinshine@126.com | Tel:0086-931-8840552 |
| --- | --- |
| Address:405 Gannan Road,Lanzhou,Gansu,China | Fax:0086-931-8840553 |

##### 9.PHYSICAL AND CHEMICAL PROPERTIES

Physical state:

Colour:

Odour:

Molecular  Formula:

Solubility:

Specific  Gravity:

Relative Vapour  Density(air=1):

Vapour    Pressure(20℃):

Flash   Point(℃):

Flammability  Limits(%):

Autoignition   Temperature(℃):

Melting   Point/Range(℃):

pH:

Solid

Yellow to Green Slight Sulfur

(CH3)2CHCH2-O-(C=S)S.Na Soluble in water.

1.17-1.18

Not available

Not available

Not available

Not available

Not available

Not applicable

Not available

10.STABILITY           AND           REACTIVITY

Reactivity:

Chemical stability:

Possibility of hazardous reactions:

Conditions to avoid:

Incompatible  materials:

Hazardous decomposition products:

Reacts with water.Reacts with acids.

Stable  under  normal  ambient  and  anticipated  storage  and  handling conditions  of  temperature  and  pressure.

Reacts exothermically with water.Heating can cause expansion or

decomposition of the material,which can lead to the containers exploding. Avoid exposure to moisture.Avoid exposure to heat,sources of ignition,    and open flame.

Incompatible  with  oxidising  agents,acids,copper,copper  alloys.

Oxides of sulfur.Carbon disulphide.Hydrogen sulfide.Carbonyl sulfide.

##### 11.TOXICOLOGICAL INFORMATION

No adverse health effects expected if the product is handled in accordance with this Safety Data Sheet and the product label.Symptoms or effects that may arise if the product is mishandled and overexposure occurs are:

Ingestion:

Swallowing may result in iritation of the gastrointestinal tract.

Eye  contact:

An  eye  iritant.

Skin contact:

Contact with skin will result in irritation.Will iberate carbon disulfide upon contact with moist skin.Carbon disulfide can be absorbed through the skin with resultant adverse   effects.

Inhalation:

Breathing in dust may result in respiratory iritation.Inhalation of carbon disulfide vapour can cause severe mood and personality disturbances including excitability,confusion

and rritability.Exposure to high vapour concentrations can result in coma.

Acute toxicity:

Oral  LD50(rat):1000  mg/kg

Chronic effects:

Not listed as carcinogenic according to IARC

12.ECOLOGICAL  INFORMATION

Ecotoxicity                                              Avoid contaminating waterways.

Gansu Winshine Metallurgy Chemical Co.,Ltd.

E-mail:gansuwinshine@126.com                                                           Tel:0086-931-8840552

Address:405 Gannan Road,Lanzhou,Gansu,China                                   Fax:0086-931-8840553

##### 13.DISPOSAL       CONSIDERATIONS

Disposal  methods:

Refer to Waste Management Authority.Dispose of material through a licensed waste contractor.Advise flammable nature.

##### 14.TRANSPORT       INFORMATION

Road and Rail Transport

Classified as Dangerous Goods by the criteria of the Chinese Dangerous Goods Code(CDG Code)for Transport

and  Rail;DANGEROUS GOODS

UN  No:

Transport Hazard Class:

Packing Group:

Proper Shipping Name or Technical Name:

Hazchem or Emergency Action Code:

3342

4.2 Spontaneously Combustible Ⅲ

XANTHATES

1Y

Marine Transport

Classified as Dangerous Goods by the criteria of the International Maritime Dangerous Goods Code (IMDG Code)for transport by sea;DANGEROUS GOODS.

UN  No:

Transport Hazard Class:

Packing Group:

Proper Shipping Name or Technical Name:

3342

4.2 Spontaneously Combustible Ⅲ

XANTHATES

IMDG EMS Fire:  IMDG  EMS  Spill

F-A S-J

Air Transport

Classified as Dangerous Goods by the criteria of the International Air Transport Association(IATA)Dangerous Goods

Regulations for transport by air;DANGEROUS GOODS.TRANSPORT PROHIBITED under the International Air Transport     Association(IATA)Dangerous Goods Regulations for transport by air in passenger aircraft only;may be transported by cargo aircraft.

UN  No:                                                         3342

Transport Hazard Class:                              4.2 Spontaneously Combustible

Packing Group:                                              IⅢ

Proper Shipping Name or                              XANTHATES

Technical Name:

Hazchem or Emergency Action                     1Y Code:

15.REGULATORY        INFORMATION

Classification:

This material is hazardous according to Safe Work China;HAZARDOUS SUBSTANCE.

Classification of the substance or mixture:

Self-heating substances and mixtures-Category 1

Acute Oral Toxicity-Category 4

Gansu Winshine Metallurgy Chemical Co.,Ltd.

| E-mail:gansuwinshine@126.com | Tel:0086-931-8840552 |
| --- | --- |
| Address:405 Gannan Road,Lanzhou,Gansu,China | Fax:0086-931-8840553 |

Acute  Dermal  Toxicity-Category  4

Skin  Irritation-Category  2

Eye  Irritation-Category  2A

Hazard  Statement(s):

H251 Self-heating;may catch fire.

H302+H312 Harmful if swallowed or in contact with skin.

H315 Causes skin irritation.

H319 Causes serious eye iritation.

Poisons   Schedule(SUSMP):                                      None allocated.

This material is listed on the Chinese Inventory of Chemical Substances(CICS).

##### 16.OTHER    INFORMATION

This safety data sheet has been prepared by YANTAI Toxicology &SDS Services

Reason(s)for  Issue:

Revised Primary SDS

Change in Fire Management Requirements Change to Transport Information

Alignment to GHS requirements

This SDS summarises to our best knowledge at the date of issue,the chemical health and safety hazards of the material  and general guidance on how to safely handle the material in the workplace.Since YANTAI cannot anticipate or control   the conditions under which the product may be used,each user must,prior to usage,assess and control therisks arising from its use of the material.

If clarification or further information is needed,the user should contact their YANTAI BILION THAI CHEMICAL CO.,LTD at the contact details on page 1.

GANSU's responsibility for the  material as sold  is subject to the terms and conditions of sale,a copy of which  is

available  upon  request.